MATLAB CODE AND RESULT:

Problem 1: Example of Nyquist bit rate calculation for a noiseless channel:

close all; clc; fs = 8000; % Sampling frequency t = 0:1/fs:1-1/fs; % Time duration cx = 1.1*sin(2*pi*100*t) + 1.3*cos(2*pi*300*t) + 1.5*sin(2*pi*2000*t); bandwidth = obw(cx,fs); % Bandwidth of the signal L=2; % Level of the signal BitRate = 2*bandwidth*log2(L)

Output:

BitRate=  3.8019e+03

Problem2: Calculation of SNR

close all; clc; %Define number of samples to take fs = 8000; % Sampling frequency f = 400; %Hz %Define signal t = 0:1/fs:1-1/fs; A = 3.0; powfund = A^2/2 s = 0.1; varnoise = s^2; signal = A*sin(2*pi*f*t); %noise noise = s*randn(size(signal)); %noisy signal noisySignal = signal + noise; SNR = snr(noisySignal) %Calculation of SNR using snr function defSNR = 10*log10(powfund/varnoise) %Calculation of SNR following the definition

Task 1: Similar task can be done considering a noisy composite signal. Suppose our composite signal is, signal = 1.5*sin(2*pi*2*t)+0.9*cos(2*pi*10*t)+1.1*sin(2*pi*20*t) + 0.13*randn(size(t)); *****Calculate the SNR value of the signal.

Solution:

close all;

clc;

% Sampling frequency

fs = 8000;

% Time vector

t = 0:1/fs:1-1/fs;

% Signal parameters

A1 = 1.5; f1 = 2;

A2 = 0.9; f2 = 10;

A3 = 1.1; f3 = 20;

% Noise parameters

s = 0.13;             % standard deviation of noise

varnoise = s^2;       % noise variance

% Composite clean signal

signal = A1*sin(2*pi*f1*t) + A2*cos(2*pi*f2*t) + A3*sin(2*pi*f3*t);

% Add noise

noise = s * randn(size(signal));

noisySignal = signal + noise;

% Signal power (theoretical)

powfund = (A1^2 + A2^2 + A3^2) / 2;

% SNR using built-in function

SNR = snr(signal, noise);

% SNR using definition

defSNR = 10*log10(powfund / varnoise);

% Display output

disp(['SNR = ', num2str(SNR)]);

disp(['defSNR = ', num2str(defSNR)]);

Output:

SNR = 21.0835

defSNR = 21.0219

Problem3: Example of Shannon capacity calculation for a noisy channel:

clc close all fs = 8000; % Sampling frequency f = 3; %Hz %Define signal t = 0:1/fs:1-1/fs; A = 2; s = 0.4; %signal x = A*sin(2*pi*f*t); %noise ns = s*randn(size(signal)); S_N_R = snr(x,ns); bandwidth = obw(x,fs); % Bandwidth of the signal %capacity C = bandwidth*log2(1+SNR) % Capacity of the channel

Output: